# Introduction to Addressing in Computer Networks

In a computer network, many devices (computers, laptops, mobiles, routers) are connected.
To communicate correctly, each device must have a unique address.

Just like a home address identifies your house, a network address identifies a device in a network.

There are two main types of addresses in networking:

- Physical Address (MAC Address)

- Logical Address (IP Address)

These two addresses work together to deliver data correctly.

# Physical Address (MAC Address)

A Physical Address is a hardware address assigned to a device's Network Interface Card (NIC).
This address is written permanently by the manufacturer.

### Key Points:

- Works at Data Link Layer (Layer 2)

- 48-bit address

- Written in Hexadecimal

- Example: A4-B3-21-6F-8C-90

### Features:

- Unique worldwide

- Never changes (unless manually modified)

- Used for communication inside the Local Area Network (LAN)

- Helps in device identification at hardware level

# Logical Address (IP Address)

A Logical Address is a software-assigned address used to identify a device on a global network like the Internet.

### Key Points:

- Works at Network Layer (Layer 3)

- Provided by Administrator / Router / DHCP

- Can change (dynamic)

- Used for device identification across multiple networks

### Example:

192.168.1.5

# Why Do We Need Physical & Logical Addresses Both?

Because:

- MAC Address helps devices communicate within the same network (LAN).

- IP Address helps devices communicate across different networks or the internet.

Both are used together in communication.

# IP Address (Internet Protocol Address)

### Definition:

An IP Address is a unique identifier assigned to devices so they can communicate on a network.

There are two versions of IP:

- IPv4 (older)

- IPv6 (newer)

# IPv4 (Internet Protocol Version 4)

### Introduction:

IPv4 is the most commonly used type of IP address in networks. It was introduced in 1980s.

### Characteristics:

- 32-bit address

- Supports around 4.3 billion devices

- Written in decimal format

- Divided into 4 parts (octets)

### Example:

192.168.0.1

### Format:

A.B.C.D
Each part contains 0–255 values.

# IPv4 Address Classes (Beginner Friendly)

IPv4 addresses are divided into five classes based on size:

| Class | Range | Used For |
| --- | --- | --- |
| A | 0–127 | Very large networks |
| B | 128–191 | Medium networks |
| C | 192–223 | Small networks |
| D | 224–239 | Multicast |
| E | 240–255 | Experimental |

### Private IPv4 Address Ranges:

- Class A: 10.0.0.0 – 10.255.255.255

- Class B: 172.16.0.0 – 172.31.255.255

- Class C: 192.168.0.0 – 192.168.255.255

# IPv6 (Internet Protocol Version 6)

### Introduction:

IPv6 was introduced because IPv4 addresses were getting over (shortage issue).

### Characteristics:

- 128-bit address

- Supports unlimited devices

- Written in hexadecimal

- Divided into 8 blocks, each separated by :

### Example:

2001:0db8:85a3:0000:0000:8a2e:0370:7334

### Why IPv6 is Better:

- Huge address space

- Faster routing

- More secure (built-in IPSec)

- Auto-configuration supported

- No need for NAT

### Short Form:

- Remove leading zeros

- Use :: for continuous zeros
Example short address:
2001:db8:85a3::8a2e:370:7334

# Difference Between IPv4 and IPv6 (Beginner Table)

| Feature | IPv4 | IPv6 |
| --- | --- | --- |
| Address Size | 32-bit | 128-bit |
| Format | Decimal | Hexadecimal |
| Example | 192.168.1.1 | 2001:db8::1 |
| Count | 4.3 billion | Unlimited |
| Security | Less secure | More secure |
| Speed | Slower | Faster |
| Header | Complex | Simple |

# Summary

- MAC Address = Physical Address = Hardware Address (Layer 2)

- IP Address = Logical Address = Internet Layer Address (Layer 3)

- IPv4 = 32-bit, limited

- IPv6 = 128-bit, unlimited

# Network ID and Host ID

An IP Address has two parts:

- Network ID (Network Portion)

- Host ID (Host Portion)

Example:
192.168.10.25

- Network ID → tells which network the device belongs to

- Host ID → tells which device in that network

The subnet mask decides how many bits are for Network ID and how many for Host ID.

## What is Network ID?

Network ID is the part of the IP address that identifies the network to which the device belongs.

It is same for all devices inside the same subnet.

### Example:

IP: 192.168.1.10
Mask: /24 → (255.255.255.0)

Network ID = 192.168.1.0

### Key Points:

- First address of subnet

- Cannot be assigned to any device

- Used by routers to forward packets

- Also called Network Address

## What is Host ID?

Host ID is the part of the IP address that identifies the specific device inside the network.

### Example:

IP: 192.168.1.10
Mask: /24

Host ID = 0.0.0.10

### Key Points:

- Unique inside a network

- Used to identify individual computers

- Range always lies between Network Address and Broadcast Address

## How to Find Network ID and Host ID?

To find Network ID:

IP Address AND Subnet Mask

To find Host ID:

IP Address MINUS Network ID

Example:
IP: 192.168.1.130
Mask: 255.255.255.0 (/24)

- Network ID = 192.168.1.0

- Host ID = 0.0.0.130

## Network ID and Host ID in Different Subnets

### Example: /26 Subnet

Subnet Mask: 255.255.255.192
Block Size: 64
Subnets:

- 0–63

- 64–127

- 128–191

- 192–255

Take IP = 192.168.1.70

Step 1 → Find its subnet:
70 belongs to 64–127 range

Step 2 → Network ID:
192.168.1.64

Step 3 → Broadcast Address:
192.168.1.127

Step 4 → Host ID:
70 – 64 = 6

So Host ID = 0.0.0.6

## Special Addresses in a Subnet

| Type | Meaning |
| --- | --- |
| Network ID | First IP of subnet, identifies network |
| Broadcast ID | Last IP of subnet, used to communicate with all hosts |
| Host Range | All usable IPs in between |

Example (Subnet: 192.168.1.64/26):

- Network ID → 192.168.1.64

- First Host → 192.168.1.65

- Last Host → 192.168.1.126

- Broadcast ID → 192.168.1.127

## Difference Between Network ID and Host ID

| Network ID | Host ID |
| --- | --- |
| Identifies the network | Identifies a device within the network |
| Same for all devices in subnet | Different for each device |
| Cannot be assigned to a host | Assigned to devices (PC, laptop, router) |
| Starts the subnet | Part of usable host space |
| Used by routers | Used by end devices |

## Why Network ID & Host ID Are Important?

- Without Network ID → routers cannot forward packets

- Without Host ID → device cannot be uniquely identified

- Essential for subnetting

- Important for secure network design

- Helps reduce broadcast and improve performance

## Super Easy Memory Trick

- Network ID = "Area name" (like a colony)

- Host ID = "House number" inside that colony

Example:
Colony: 192.168.1.
House number: 10

So 192.168.1.10 = House No. 10 in that colony.

## Summary

- IP address = Network ID + Host ID

- Network ID = Identifies the network

- Host ID = Identifies the device

- Subnet mask decides how network and host bits are split

- Network ID is always the first address

- Broadcast ID is always the last address

- Hosts lie in between them

# What is Subnetting?

Subnetting means dividing one large network into smaller networks.

Example:
If you have a big network 192.168.1.0, you can divide it into small groups like:

- 192.168.1.0

- 192.168.1.64

- 192.168.1.128

- 192.168.1.192

This helps in:

- Better security

- Better management

- Less broadcast traffic

- Faster network

## Why Subnetting is Needed?

Subnetting is used when:

- You want to separate departments (like Accounts, HR, IT).

- You want security, so each department has its own network.

- You want to reduce broadcast traffic.

- You want to efficiently use IP addresses.

Without subnetting = One big crowd
With subnetting = Divided into smaller, manageable groups

## Important Terms in Subnetting

## 1. Network Address

The starting address of a network.
Example: 192.168.1.0

## 2. Broadcast Address

The last address of a network.
Used to send data to all devices in the subnet.
Example: 192.168.1.63

## 3. Host Address

Addresses used by devices (computers, mobiles).
Example: 192.168.1.1 – 192.168.1.62

## 4. Subnet Mask

A number that tells how many bits are used for the network.

Example:
255.255.255.0 → 24 bits for network → /24
255.255.255.128 → 25 bits for network → /25

## Subnet Mask Basics

Subnet mask divides the IP address into:

- Network part

- Host part

Example:
255.255.255.0 → /24

Binary:
11111111.11111111.11111111.00000000
(24 ones → network bits)

## How Subnetting Works (Very Simple Explanation)

Original network: 192.168.1.0/24

If you divide it into 2 subnets:

- New mask becomes /25

- Each network gets 128 addresses

- Subnets formed:

### Subnet 1:

Network: 192.168.1.0
Broadcast: 192.168.1.127
Hosts: 1–126

### Subnet 2:

Network: 192.168.1.128
Broadcast: 192.168.1.255
Hosts: 129–254

## Number of Hosts Formula

### Formula:

Hosts = (2^remaining bits) – 2

We subtract 2 because:

- 1 network address

- 1 broadcast address

Example for /26:
/26 means 32 - 26 = 6 host bits
Hosts = 2⁶ – 2 = 64 – 2 = 62 hosts

## Finding Subnet Size (Block Size)

Easy method:
256 – last subnet mask value

Example:
Subnet mask: 255.255.255.192
Last part: 192
Block size = 256 – 192 = 64

So networks will be:
0, 64, 128, 192

## Example 1: Subnet 192.168.10.0/26

Step 1: /26 → mask 255.255.255.192
Step 2: Block size = 256 – 192 = 64
Step 3: Subnets:

- 192.168.10.0
Broadcast: 63
Hosts: 1–62

- 192.168.10.64
Broadcast: 127
Hosts: 65–126

- 192.168.10.128
Broadcast: 191
Hosts: 129–190

- 192.168.10.192
Broadcast: 255
Hosts: 193–254

## Example 2: How many hosts in /27?

/27 → 32 - 27 = 5 host bits
2⁵ – 2 = 32 – 2 = 30 hosts

## CIDR Notation (Super Simple)

CIDR means writing subnet mask in slash format.

Examples:

- /24 = 255.255.255.0

- /25 = 255.255.255.128

- /26 = 255.255.255.192

- /27 = 255.255.255.224

- /28 = 255.255.255.240

Think: bigger number = smaller subnet = fewer hosts

## Summary (Beginner Level)

- Subnetting = dividing one network into smaller networks.

- Helps security, speed, and organization.

- Subnet mask decides network size.

- Block size = 256 – subnet mask last value.

- Hosts = (2^host bits) – 2.

- Smaller subnets = fewer host devices.

# Subnetting Practice Questions (with Answers)

## Q1. Find the Network ID of IP: 192.168.10.25/24

Solution:
/24 → 255.255.255.0
Network ID = 192.168.10.0

## Q2. Find Broadcast Address of 192.168.5.90/26

/26 → Mask 255.255.255.192
Block Size = 256 − 192 = 64
Subnet ranges: 0–63, 64–127, 128–191, 192–255

90 lies in 64–127
Network ID = 192.168.5.64
Broadcast = 192.168.5.127

## Q3. How many hosts in a /27 network?

Host bits = 32 − 27 = 5
Hosts = 2⁵ − 2 = 30 hosts

## Q4. Find first host of 10.0.8.0/22

/22 = Mask → 255.255.252.0
Block size = 256 − 252 = 4
Subnets: 0, 4, 8, 12...

Given network: 10.0.8.0
First host = 10.0.8.1

## Q5. Find last host of 10.0.12.0/22

Same block 4
Subnet ranges: 8.0–11.255, 12.0–15.255

Last host = 10.0.15.254

## Q6. IP 172.16.50.100/20 → Find Network ID

/20 mask = 255.255.240.0
Block = 256 − 240 = 16
50 lies in 48–63

Network = 172.16.48.0

## Q7. How many subnets in /26 if original is /24?

Borrowed bits = 26 − 24 = 2
Subnets = 2² = 4 subnets

## Q8. IP 192.168.1.200/25 → Find Broadcast

/25 mask = 255.255.255.128
Block = 256 − 128 = 128
Subnet ranges: 0–127, 128–255

Broadcast = 192.168.1.255

## Q9. Find host range of 172.16.0.0/23

/23 → Mask 255.255.254.0
Block = 256 − 254 = 2
Subnets: 0–1, 2–3, etc.

Network = 172.16.0.0
Broadcast: 172.16.1.255

Hosts: 172.16.0.1 → 172.16.1.254

## Q10. IP 192.168.100.77/28 → Find Network ID & Broadcast

/28 → Mask 255.255.255.240
Block = 256 − 240 = 16
Subnets: 0,16,32,48,64,80...

77 lies in 64–79

Network ID = 192.168.100.64
Broadcast = 192.168.100.79

## 2. Subnet Mask Table for Exam (Easy Memory Table)

| CIDR | Subnet Mask | Block Size | Usable Hosts |
| --- | --- | --- | --- |
| /24 | 255.255.255.0 | 256 | 254 |
| /25 | 255.255.255.128 | 128 | 126 |
| /26 | 255.255.255.192 | 64 | 62 |
| /27 | 255.255.255.224 | 32 | 30 |
| /28 | 255.255.255.240 | 16 | 14 |
| /29 | 255.255.255.248 | 8 | 6 |
| /30 | 255.255.255.252 | 4 | 2 |
| /31 | 255.255.255.254 | 2 | 0 (Used for P2P Links) |
| /32 | 255.255.255.255 | 1 | 0 (Single host label) |

Tip:
Bigger CIDR number → smaller network → fewer hosts.

## 3. Network ID & Host ID Exercises

Try these yourself (answers also included below).

## Exercise 1

IP: 192.168.20.55/26
Find:

- Network ID

- Broadcast

- Host Range

- Host ID

### Solution:

Block = 64
Subnets: 0–63, 64–127, 128–191, 192–255
55 → 0–63 subnet

- Network: 192.168.20.0

- Broadcast: 192.168.20.63

- Hosts: 1–62

- Host ID = 55 − 0 = 55

## Exercise 2

IP: 10.10.17.200/20
Find Network ID.

### Solution:

/20 → 255.255.240.0
Block = 16
17 is in 16–31
Network = 10.10.16.0

## Exercise 3

IP: 172.31.46.8/23
Find Host Range.

### Solution:

/23 → 255.255.254.0
Block = 2
46 is in 46–47

Network = 172.31.46.0
Broadcast = 172.31.47.255
Hosts = 172.31.46.1 → 172.31.47.254

## Exercise 4

IP: 192.168.1.150/25
Find Host ID.

### Solution:

/25 → 128 block
150 lies in 128–255
Network = 128
Host ID = 150 − 128 = 22

## Exercise 5

How many hosts in /21?

### Solution:

Host bits = 32 − 21 = 11
Hosts = 2¹¹ − 2 = 2046 hosts